Electric Longboard Mark II

Open-Source Project by Daniil Andreev

Oct. 16, 2017

Motivation

The reasoning behind this project it is a natural progression of my interest in longboard and previous mechanics experience, as well as making a resume that I can literally ride to an interview. However, the initiating thought was; I want build something so outrageous it exists only in a Neal Stevenson universe.

The project started during the end of my first year as an undergrad at Knox College. I had looked at the commercially available Boosted Board and figured I could make something way more powerful.

Having no training in electronics, I spent the rest of the year attempting smaller projects that would help me build the skill required to attempt an electric vehicle. 3 years later I am just starting to see the finish line.

Mechanical Design and Model

After a year break in the building process I set out to reimagine the mechanical superstructure.

Completed

2015-2016

| V4 Carbon Fiber Deck | V2 10W RGBW Headlights, Backlights |
| --- | --- |
| 3 failed designs | 4 x ILD6150 LED Drivers Atmel 328 Micro 12-65 operating voltage |

| Daisy chainable single cell BMS | V1.5 Truck and Motor Assembly |
| --- | --- |
| Attiny 85 Micro Isolated I2C channel Onboard boost converter | 8kW Drive train 2 x Turnigy 7464 BLDC Motors 2 x Kegel Orangatang 80mm Wheels CNCed and turned at Redwood City Techshop. |

In the Pipeline:

2017-

| Battery Pack | Carbon fiber deck |
| --- | --- |
| 64 LiFePO4 cells to deliver 200A @ 60V continuous | Molds CNCed Carbon fiber bought |
| Sensor Module | 1.5kW charger |
| Raspberry Pi central module GPS Altimeter Design and development in collaboration with Sam Ragsdale | Tested at 900W without water-cooling Compatible with 240V for 3kW+ charging Design and development in collaboration with Anton Zaytsev |

In the Near Future

Headlight, Backlight enclosure

Integrated battery management system

Water-resistant cover

Remote-control system

2 x V2 trucks and motor mounts for the full 16kW drive train.

Learning Experience

This project helped me become a full stack developer. Having the opportunity to design and manufacture the whole project from scratch taught me to be very considerate of my design choices; making sure that the hardware is compatible with the electronics and software, striking the balance between aesthetic design and ease of manufacture. To make this board a reality I had to crawl along the entire industrial process, learning the properties of plastics, metals, composites and their respective postprocessing techniques. I learned to do the gritty work as well, to not only design and build, but talk to PCB manufacturers, spend hours on the phone with hardware suppliers, and read endless datasheets, weigh the properties of different chipsets and picking products that integrate with the full stack development process.

The top five skills that I this project taught me:

CAD 			– Solidworks, Eagle

Electronics		– Circuit Design, PCB Design, Manufacture

C programming 	– Firmware development

Manufacturing 	– CNC, Lathe, Mill, Carbon Fiber Molding

Self-Teaching	– The ability to research, learn and use any new skill or       _ information that I need to push the project along

In Conclusion

Experiencing the width of the engineering process has inspired me to join a EE master program and join the hardware world to make products that have a physical impact.

Components not designed or manufacture by project team

VESC – BLDC motor speed controller

Turnigy 6374 BLDC motors

Nyko Wii nunchuck – repurposed for the control of the ESCs

Kegel Orangatang 80mm Wheels

More Info

Hackaday Project Page

Github